# Task Title: Millwright Maintenance Procedure

# OALCF Cover Sheet – Practitioner Copy

Learner Name: __________________________________________

Date Started: ___________________________________________

Date Completed:  ________________________________________

| Goal Path: | Employment | Apprenticeship |
| --- | --- | --- |
| Secondary School | Post Secondary | Independence |

Successful Completion:  Yes  	No

Task Description:

The learner will scan a document and follow written instructions to perform tasks related to using a procedure in a workplace.

Main Competency/Task Group/Level Indicator:

- Find and Use Information/Read continuous text/A1.2

Materials Required:

- Pen/pencil and paper

Learner Information

Millwrights follow a procedure to perform required maintenance in the workplace. This helps them to ensure the correct tools, measurements and safety procedures are followed.

Scan the Millwright Maintenance Procedure: Changing and Inspecting Gears instructions.

Millwright Maintenance Procedure: Changing and Inspecting Gears

Step 1: Safety Precautions

- Power off the machine and follow lockout/tagout (LOTO) procedures to ensure safety.

- Wear proper PPE (gloves, safety glasses, steel-toe boots).

- Verify that all moving parts are fully stopped before proceeding.

Step 2: Remove the Gear Assembly

- Use a 13mm wrench to remove the housing cover and expose the gears.

- Note the orientation of the gears before removal (take a picture if necessary).

- Use a gear puller to remove the gear carefully, avoiding damage to surrounding components.

- Place the removed gear on a clean surface for inspection.

Step 3: Inspect the Gear

- Check for wear and damage:

  - Look for pitting, cracks, broken teeth, or unusual wear patterns.

  - If damage is found, the gear must be replaced.

- Measure the gear thickness using a digital caliper:

  - Compare the reading with the specification in the manual (e.g., 12.5 mm ± 0.1 mm).

  - If out of range, use a new gear.

- Check gear backlash (clearance between teeth) using a feeler gauge:

  - Compare the measurement with the acceptable range (0.15 mm ± 0.05 mm).

  - If out of tolerance, adjust the gear positioning during installation.

Step 4: Install the New Gear

- Clean the mounting surface using a degreaser and a clean cloth.

- Apply a thin layer of lubricant to the shaft and gear teeth.

- Position the new gear correctly, ensuring proper alignment with existing components.

- Secure the gear with bolts and use a torque wrench to tighten according to specifications.

Step 5: Final Alignment and Testing

- Use a dial indicator with a digital display to check gear runout (misalignment).

  - The reading must not exceed 0.02 mm.

  - If out of tolerance, adjust the gear position.

- Manually rotate the gear to ensure smooth movement.

- Reinstall the housing cover and secure it with the 13mm wrench.

Step 6: Documentation and Cleanup

- Record the following in the online maintenance log:

  - Gear replacement details (measurements, clearances, torque settings).

  - Condition of the old gear (reason for replacement).

  - Any additional notes or issues encountered.

- Remove tools and debris from the work area.

- Remove lockout/tagout devices and power on the machine.

- Observe the machine running to ensure proper gear function.

# Work Sheet

Task 1: List two safety precautions to follow.

Answer:

______________________________________________________

Task 2: Which tool is used to remove the gear assembly?

Answer:

______________________________________________________

Task 3: The gear has pitting and a broken tooth. What should the millwright do?

Answer:

______________________________________________________

Task 4: The gear backlash clearance measures 0.22 mm. What should the millwright do?

Answer:

______________________________________________________

Task 5: In the Final Alignment and Testing what does the dial indicator measure?

Answer:

______________________________________________________

Task 6: List two items that must be recorded in the online maintenance log.

Answer:

______________________________________________________

# Answers

Task 1: List two safety precautions to follow.

Answer: Any two of the following are acceptable

- Power off the machine and follow lockout/tagout (LOTO) procedures to ensure safety.

- Wear proper PPE (gloves, safety glasses, steel-toe boots).

- Verify that all moving parts are fully stopped before proceeding.

Task 2: Which tool is used to remove the gear assembly?

Answer: A 13 mm wrench

Task 3: The gear has pitting and a broken tooth. What should the millwright do?

Answer: Replace the gear

Task 4: The gear backlash clearance measures 0.22 mm. What should the millwright do?

Answer: Adjust the gear positioning during installation

Task 5: In the Final Alignment and Testing what does the dial indicator measure?

Answer: Checks the gear runout or misalignment

Task 6: List two items that must be recorded in the online maintenance log.

Answer: Any two of the following are acceptable

- Gear replacement details (measurements, clearances, torque settings).

- Condition of the old gear (reason for replacement).

- Any additional notes or issues encountered.

# Performance Descriptors

| Levels | Performance Descriptors | Needs Work | Completes task with support from practitioner | Completes task independently |
| --- | --- | --- | --- | --- |
| A1.2 | Scans text to locate information |  |  |  |
|  | Locates multiple pieces of information in simple texts |  |  |  |
|  | Reads more complex texts to locate a single piece of information |  |  |  |
|  | Makes low-level inferences |  |  |  |

This task: Was successfully completed        Needs to be tried again

Learner Comments:

Instructor (print):					Learner (print):

_______________________                           _______________________